# TECHNICAL PROJECT REPORT

# Title of Invention / Project: Automatic Alert Via Bluetooth With Buzzer

Home monitoring system

# Team Members / Inventors:

| S. No. | Name | Department | Designation | Mobile | E-Mail |
| --- | --- | --- | --- | --- | --- |
| 1. | Raghav Goel | CSE | Student | 8168127644 | raghavgoel847@gmail.com |
| 2. | Varnika Sharma | CSE | Student | 9654315567 | varnika.sharma1801@gmail.com |
| 3. | Vanshita Manral | CSE | Student | 8146858558 | manral.14vanshita@gmail.com |
| 4. | Vanshika Sanan | CSE | Student | 7831084984 | sanan.vanshika@gmail.com |
| 5. | Khushal Thakur | ECE | Mentor | 9646030764 | khushal.thakur@cumail.in |
| 6. | Anshul Sharma | ECE | Mentor | 9478697475 | anshulsharma.ece@cumail.in |
| 7. | Kiran Jot Singh | ECE | Mentor | 9463909689 | kiranjotsingh.ece@cumal.in |
| 8. | Divneet Singh Kapoor | ECE | Mentor | 9878422653 | divneet.ece@cumail.in |

Section – 1 (IPR Related)

# Abstract:

- Problem your project is solving?

Security is one of the major concerns in everyone’s life, and therefore surveillance is important.

We can check or monitor our house on a full-time basis, we can check whether someone is lurking around our house.

- How are you solving that?

We have built a smart door sensing device using Arduino which can easily sense and detect a person coming near the door in certain detection distance and will ring the buzzer in the given range. Our system will also send an intruder alert to a connected mobile phone via Bluetooth, which can make the person alert that someone is there at his\her place.

- Additional modifications that can cater to improved solution.

- For the additional modification we can attach a camera/ surveillance which will send photos of our guest.

- We can also install an intercommunication system which will enable us to identify and record the voice of the intruder.

- We can give a notification or alert with the help of wireless WIFI (esp2866) module via mail system.

# Existing state-of-the-art and Drawbacks in existing state-of-the-art

| S. No. | Existing state of art | Drawbacks in existing state of art |
| --- | --- | --- |
| 1 | # Suspicious individual detector # https://patents.google.com/patent/KR101492506B1/en?q=home&q=monitoring&q=security&q=system&q=via&q=bluetooth&q=alert&q=buzzer&oq=home+monitoring+security+system+via+bluetooth+alert+and+buzzer | It gives false alerts many times and is expensive. |
| 2 | Monitoring device, monitoring system, monitoring method and program. https://patents.google.com/patent/KR101492506B1/en?q=home&q=monitoring&q=security&q=system&q=via&q=bluetooth&q=alert&q=buzzer&oq=home+monitoring+security+system+via+bluetooth+alert+and+buzzer | The response time is slower comparatively. |

# Novel/Additional modifications that you can propose to improve upon drawbacks

- Feature 1

We can use camera or surveillance instead of alerts and install a sound system to hear voices.

- Feature 2

We can use wi-fi module instead of bluetooth module, as bluetooth is short ranged.

# Advantages

- Adv 1

Its is deterrent to the crime.

- Adv 2

It allows to have a remote access to your home.

- Adv 3

Low maintenance cost and can be easily executed.

- Adv 4

Prevents unknown intrusion attempts.

# Block Diagram

SMART PHONE

Section – 2

(Real Project)

# Materials

- Battery with cap

Description:

Voltage= 9 volts

Non- rechargeable

Used for- toys, watch, etc

- Arduino UNO

Microcontroller-ATmega328

Operating voltage- 5V

Input voltage- 7-12V

Input voltage- 6-20V

Digital I/O pins 14 (6 of which provide PWM)

Analog Input pins- 6

- Breadboard

Description:

400 tie points

2 Power Lanes, Tie-pints 100

1 Double-strip, Tie-points 300

Size- 8.2* 5.5* 0.85 cm

Voltage/current- 300V/3-5A

- Ultrasonic Sensor

Working voltage: 5V (dc)

Static current: less than 2 mA

Output signal: electrical frequency signal,

High level 5V, low level 0V

Sensor angle: less than 15 degrees

Detection distance: 2cm-450cm

- Alarm/Buzzer

B20 buzzer

Piezoelectric 3-12V

Used in: bikes, electronics circuit

- Jumper Wires

Wire length: 20cm

The male ends meant for insertion into standard

0.1inch female socket

The female ends are meant for insertion onto standard

0.1inch male headers

- Bluetooth module- HC-05

# Circuit Diagram

# Steps of Circuit Completion:

The pictures of the different stages  in the circuit are as follows –

Bluetooth hc-05

- The Buzzer long leg (+) connect to the Arduino Board Digital 11

- The Buzzer short leg (-) connect to the Arduino Board GND

- The HC-SR04 Ultrasonic Module has 4 pins, Ground, VCC, Trig and Echo. The Ground and the VCC pins of the module needs to be connected to the Ground and the 5 volts pins on the Arduino Board respectively and the trig and echo pins to any Digital I/O pin on the Arduino Board.

- The HC-SR04 sensor attach to the Breadboard

- The Sensor VCC connect to the Arduino Board +5V

- The Sensor GND connect to the Arduino Board GND

- The Sensor Trig connect to the Arduino Board Digital I/O 9

- The Sensor Echo connect to the Arduino Board Digital I/O 10

Bluetooth:

- TX OF BLUETOOTH TO RX OF ARDUINO

- RX OF BLUETOOTH TO TX OF ARDUINO

- VCC TO BATTERY

- GND TO GND

The picture of Phase 1 of the circuit –

First Phase of Connection

# Second Phase of Connection

Final Phase of the circuits

# Program Code

https://github.com/vanshitamanral/home-monitoring-system